# Stage 1 Participant Submission - Security of Drones (Objective 2)

| Team ID | PF-4029 |
| --- | --- |
| Team Name | Persistent Formation |
| Institution / Organization | Independent Research Team |
| Team Leader | Sumit Saraswat |
| Team Members | Sumit Saraswat, Tanmay Kaushal |
| Faculty / Industry Mentor, if any | None |
| Email Address | sumitsaraswat362@gmail.com |
| Contact Number | +91 9999999999 |
| Proposed Design Name | AEGIS — Autonomous Embedded Guardian for Intrusion in Swarms |
| Date of Submission | 27 September 2026 |

## 1. Executive Summary

AEGIS is a novel, production-grade Drone Intrusion Detection System (IDS) built on a Cyber-Physical Digital Twin architecture. Unlike conventional IT-oriented IDS solutions that apply static rules to network packets, AEGIS fuses aerospace Guidance, Navigation, and Control (GNC) principles with machine learning to monitor, detect, and classify cyber-threats against MAVLink-based UAVs in real-time.

The key innovation is a Kinematic Extended Kalman Filter (EKF) that maintains a running physics-based prediction of the UAV's 6-DOF state (position and velocity). Sensor readings are compared against this prediction using the Mahalanobis Distance in a Chi-Squared statistical test. When the sensor reading becomes statistically incompatible with the UAV's physical dynamics, AEGIS raises a CRITICAL alert.

Key Deliverables include a Tri-engine detection pipeline, a Tamper-evident Cryptographic Chain Logger, a Live Vercel dashboard with WebSocket backend, and a 400-run Monte Carlo validation.

## 2. Understanding of the Problem

The MAVLink protocol, the dominant UAV communication protocol, has no native authentication, encryption, or replay protection. Existing network IDS (like Snort) are designed for TCP/IP and have no understanding of UAV physics. A drone accelerating rapidly is normal, but teleporting 200 meters in 0.1 seconds is a GPS spoof. A rule-based system alone generates false positives. The only correct solution is a physics-aware IDS that detects attacks based on kinematic violations.

## 3. Proposed Drone Forensic Toolkit / IDS

AEGIS consists of a Tri-Engine Architecture:

1. EKF Digital Twin (Primary Innovation): Tracks the UAV state vector [x, y, z, vx, vy, vz]. On receiving a GPS packet, the innovation residual and covariance are computed. If the Mahalanobis distance d^2 > 16.27 (99.9% confidence), it flags a NavIC/GPS spoof.

2. Isolation Forest (ML Anomaly Detector): A 12-dimensional feature vector is extracted from a 2-second sliding window. An Isolation Forest scores live packets to catch novel zero-day anomalies.

3. Deterministic Rule Engine: Fast O(1) checks for Command Injections (R5), DoS Floods (R4), and Replay attacks (R6).

Forensic capabilities are handled by a Cryptographic Chain Logger that hashes every event with SHA-256 to ensure tamper-evident evidence.

## 4. Data Analysis & Automated Forensic Reporting

When an anomaly is detected, AEGIS logs the precise Mahalanobis variance and triggered rules. The evidence is stored in a JSONL chain-of-custody format where each entry hashes the previous entry. A live WebSocket dashboard visualizes the EKF thresholds and threat radar in real-time, allowing operators to immediately isolate compromised drones.

## 5. Development Plan

1. Core simulation engine + attack injector (Complete)
2. EKF Digital Twin & Isolation Forest (Complete)
3. Real-Time WebSocket Dashboard (Complete)
4. 400-run Monte Carlo Validation (Complete)
5. Hardware integration on Pixhawk + RPi4 (Oct-Nov 2026)
6. Live demo at Techfest (Dec 2026)

## 6. Validation & Testing Methodology

AEGIS was statistically validated across 400 simulation runs. The following test cases demonstrate the system's accuracy.

| Test ID | Test Description | Input Evidence | Expected Result | Success Criteria |
| --- | --- | --- | --- | --- |
| TC-01 | GPS Spoofing detection via EKF | MAVLink stream with lat/lon jump at t=15s | EKF alert within 1 packet | d^2 > 16.27, CRITICAL alert emitted |
| TC-02 | Command injection from rogue GCS | COMMAND_LONG (LAND) from system_id=99 | CRITICAL alert R5+R7 | Alert severity = CRITICAL, sys_id flagged |
| TC-03 | Denial-of-Service heartbeat flood | 80 HEARTBEAT pkts/sec burst | HIGH alert R4 | Rate > threshold, alert in < 100ms |
| TC-04 | Replay attack detection | Stale-timestamp GPS packet reinjected | HIGH alert R6 | Timestamp delta > 2.0s detected |
| TC-05 | Clean flight (no false positive) | 120s nominal flight, no attacks | No alerts generated | 0 alerts, FPR = 0% |
| TC-06 | Combined GPS spoof + Command injection | Simultaneous dual-vector attack | Both CRITICAL alerts | Both detection engines fire |